This document contains sii3512ide-specific documentation information.

Please see also :

- ReleaseNotes/sii3512ide_dev_relnotes

How to use

==========

- To use the driver, just drop a sii3512-based SATA board

into any PCI slot, connect it to a SATA drive (a real one,

not a PATA with a SATA converter, if you want a real SATA

setup), and add the driver to your kicklayout.

- This driver can work happily aside the other OS4 disk

drivers.

- The 3512 does not directly support a harddisk activity LED.

- The SiI3512 chip handles only 2 SATA drives

(harddisks or ATAPI). Both are seen as 'masters', and will be

respectively units 0 and 1. If you

want to spare timeouting when trying to configure the

nonexistent drives, please setenv sii3512ide_conf to match

your config (see below).

From version 51.48 on, the drivers do not handle the nonexistant

slave devices. Thus, the driver's uboot envvars go from 4 chars

down to 2 chars.

From version 51.49 on, the driver supports up to 4 PCI chips / boards.

I - Configuring sii3512ide through UBOOT envvars

================================================

Currently sii3512ide is configurable through the following UBOOT envvars :

- sii3512ide_verbose

Legal values are : set to anything makes the driver verbose at boot time.

Default if unset : driver silent.

- sii3512ide_maxbus

Legal values are :  0 (no SATA interface at all),

1 (only primary SATA interface), and

2 (both SATA interfaces)

Default : if you don't set this envvar, sii3512ide will use both SATA interfaces.

- sii3512ide_timeout

Legal values are between 1 and 30 (seconds). The latter is the recommended

value from the ATA(PI) spec.

Default : if you don't set this envvar, sii3512ide will use a 20 second timeout.

- sii3512ide_conf

Compose 2 chars (prim SATA, sec SATA) using :

0 : nothing

1 : harddisk

2 : dvd/cdrom reader or burner

Default : if you don't set this envvar, sii3512ide will scan the IDE buses and

try to determine the configuration by itself.

- sii3512ide_xfer

Allows to define the xfer mode which will be used for each drive. Compose

2 chars (prim SATA, sec SATA) using :

0 - Automatic PIO (driver will use the best PIO mode

that the drive claims to support)

a - PIO 0  (  3 MB/s, modeid 8)

b - PIO 1  (  5 MB/s, modeid 9)

c - PIO 2  (  8 MB/s, modeid 10)

d - PIO 3  ( 11 MB/s, modeid 11)

e - PIO 4  ( 16 MB/s, modeid 12)

A - UDMA 0 ( 16 MB/s, modeid 64)

B - UDMA 1 ( 25 MB/s, modeid 65)

C - UDMA 2 ( 33 MB/s, modeid 66)

D - UDMA 3 ( 44 MB/s, modeid 67)

E - UDMA 4 ( 66 MB/s, modeid 68)

F - UDMA 5 (100 MB/s, modeid 69)

G - UDMA 6 (133 MB/s, modeid 70)

NB : sii3512ide will fall back to the best mode that the drive claims to support

in case you ask for an unsupported mode.

Default : if you don't set this envvar, sii3512ide will use the best PIO mode

supported by the drive.

- sii3512ide_irq

Compose 2 chars (prim SATA, sec SATA) using :

1 : use IRQs

any other char : don't use IRQs

Default : if you don't set this envvar, sii3512ide WILL use interrupts.

Using interrupts relieve the CPU in all cases (PIO : approx 20%, and UDMA

approx 95%), but it also has an incidence on xfer speed : 10 to 20% lower

for PIO, 10 to 20% faster for UDMA. This is normal (in PIO, it involves 2

exec reschedulings, and in UDMA, it releases the VIA bus).

EXAMPLE : to ide timeout to 5 seconds,

force prim SATA as harddisk, sec SATA as cd, force prim SATA to

udma33 and let sii3512ide select the best PIO xfer

mode for sec SATA, enter the following commands at the UBOOT

prompt :

setenv sii3512ide_timeout 5

setenv sii3512ide_conf 12

setenv sii3512ide_xfer C0

saveenv

Then reboot. Don't forget the 'saveenv', as sii3512ide uses nonvolatile.library,

which only accesses the envvars stored in nvram.

The driver supports up to 4 sii3512ide PCI boards / chips. For the 2nd, 3rd etc.

chips, add '.2', '.3' etc. to envvar names. I.e., setenv sii3512ide_conf.2 12

II - Notes

==========

- Since 51.48, this driver runs one task per IDE channel (or port

or chain or interface),  i.e. one task for the primary IDE chain

and one task for the secondary IDE chain. Those tasks operate in

parallel, which means that the driver can write to a CD on the

secondary channel while reading from a harddisk on the primary

channel. So from this version on, it makes sense to dispatch your

IDE peripherals accordingly.

- This configurability feature relies on two kickstart modules from Ross

Vumbaca that MUST be listed in your kicklayout : nonvolatile.library.kmod

50.2+ and nvram.resource.kmod. If those modules are not present, sii3512ide will

revert to default values.

- These envvars HAVE to be saved ('saveenv' after modifying them), because

they are read directly from the nvram.

- To delete an envvar, just define a blank content & then saveenv (ie

'setenv sii3512ide_conf' and then 'saveenv' will delete this envvar).

- Using sii3512ide_conf (properly set !) saves need for timeouts for drive

detection at boot time.

- It looks like each CD/CDR/DVD/DVDR drive reports its own status codes

when a blank media is inserted. Thus, OS4 will report different info

on different systems when a blank CDR is inserted.

- Colin Wenzel reports that the IDE ZIP works.

- When you grab a boot log & send it to me, please use a nondebug kernel.

- If you experience VERY SLOW harddisk writes with FFS, then you probably

have a drive that does not execute properly a FLUSH_CACHE command

(sii3512ide checks that at init time). Thus, sii3512ide disables the drive's

write cache. Thus, your drive really crawls in writing.

This does not show for the same drive with other drivers under other

os'es, because they don't check if the drive can execute a FLUSH_CACHE,

and just leave the write cache ON.

Under Linux or AOS3.x, this can *look* fast & reliable, but this *is* fast

and not reliable.

Some filesystems depend on proper execution of FLUSH_CACHE, when *they*

decide it, not when the drive decides. Thus, those fs'es can appear to be

working, but they are not really reliable. SFS is one of those.

Thus, it was decided (long ago) by the os4 dev team (mainly js & myself ;),

to disable the write cache if the drive cannot properly execute a

FLUSH_CACHE.

The consequence is that under those circumstances, FFS really crawls at

write, but you can move to the latest SFS + its diskcache.library,

which works around this problem while (mostly) staying on the safe side.

Make sure to create your SFS partition this way :

- 500 buffers

- blocksize set to 512

- maxtransfer set to 7FFFFFFF

- mask set to FFFFFFFE

III - Fine Art of drive partitioning

===================================

A few things you must know :

- You'll get decent performance from any FS only with buffers >= 500

- You'll get decent performance from FFS only with blocksize >= 2048

- You'll get the best performance from SFS only with blocksize = 512

- SFS is way faster than FFS in its default (non-caching) behaviour,

but FFS is said to be safer.

- SFS + its diskcache.library is even faster.

- There is a cache plugin for FFS, and you can also set FFS in

writeback cache mode. Using both things make it waaaaay faster

(almost comparable to SFS). The FFS cache plugin is 'fs_cache_plugin',

you will find it somewhere on your OS4 CD, just 'run fs_cache_plugin DH0:'

for each FFS partition you wish to run with cache.

- The command I use to bench r/w perf is scsispeed, found in the

latest diskspeed archive from Aminet, and also in an OS4 native

form on the OS4 beta server (quick port by myself, dropped the

m68k CPU usage measurement code).