lspci(8)                       The PCI Utilities                      lspci(8)

NAME

lspci - list all PCI devices

SYNOPSIS

lspci [options]

DESCRIPTION

lspci  is  a  utility for displaying information about all PCI buses in

the system and all devices connected to them.

By default, it shows a brief list of devices. Use the options described

below  to  request  either a more verbose output or output intended for

parsing by other programs.

If you are going to report bugs in  PCI  device  drivers  or  in  lspci

itself,  please  include  output  of "lspci -vvx" or even better "lspci

-vvxxx" (however, see below for possible caveats).

Some parts of the output, especially in the highly  verbose  modes,  is

probably  intelligible  only  to experienced PCI hackers. For the exact

definitions of the fields, please consult either the PCI specifications

or the header.h and /usr/include/linux/pci.h include files.

Access  to  some  parts of the PCI configuration space is restricted to

root on many operating systems, so the features of lspci  available  to

normal  users  are limited. However, lspci tries its best to display as

much as available and mark all other information with  <access  denied>

text.

OPTIONS

-v     Be verbose and display detailed information about all devices.

-vv    Be  very  verbose  and display more details. This level includes

everything deemed useful.

-vvv   Be even more verbose and  display  everything  we  are  able  to

parse,  even  if it doesn't look interesting at all (e.g., unde-

fined memory regions).

-n     Show PCI vendor and device codes as numbers instead  of  looking

them up in the PCI ID list.

-x     Show  hexadecimal dump of the standard part of the configuration

space (the first 64 bytes or 128 bytes for CardBus bridges).

-xxx   Show hexadecimal dump of the whole PCI configuration  space.  It

is  available only to root as several PCI devices crash when you

try to read some parts of the config space (this behavior proba-

bly  doesn't  violate  the  PCI standard, but it's at least very

stupid). However, such devices are rare, so  you  needn't  worry

much.

-xxxx  Show hexadecimal dump of the extended (4096-byte) PCI configura-

tion space available on PCI-X 2.0 and PCI Express buses.

-b     Bus-centric view. Show all IRQ numbers and addresses as seen  by

the cards on the PCI bus instead of as seen by the kernel.

-t     Show  a tree-like diagram containing all buses, bridges, devices

and connections between them.

-s [[[[<domain>]:]<bus>]:][<slot>][.[<func>]]

Show only devices in the specified domain (in case your  machine

has  several  host  bridges,  they can either share a common bus

number space or each of them can address a  PCI  domain  of  its

own;  domains  are numbered from 0 to ffff), bus (0 to ff), slot

(0 to 1f) and function (0 to 7).  Each component of  the  device

address  can be omitted or set to "*", both meaning "any value".

All numbers are hexadecimal.  E.g., "0:" means  all  devices  on

bus  0,  "0"  means  all functions of device 0 on any bus, "0.3"

selects third function of device 0 on all buses and  ".4"  shows

only the fourth function of each device.

-d [<vendor>]:[<device>]

Show only devices with specified vendor and device ID. Both ID's

are given in hexadecimal and may be omitted  or  given  as  "*",

both meaning "any value".

-i <file>

Use    <file>    as    the    PCI    ID    list    instead    of

/usr/local/share/pci.ids.

-m     Dump PCI device data in machine readable form (both  normal  and

verbose format supported) for easy parsing by scripts.

-M     Invoke  bus  mapping  mode which performs a thorough scan of all

PCI devices, including those behind misconfigured  bridges  etc.

This  option  is  available only to root and it gives meaningful

results only if combined with direct hardware access mode  (oth-

erwise the results are identical to normal listing modes, modulo

bugs in lspci). Please note that the bus mapper doesn't  support

PCI domains and scans only domain 0.

--version

Shows lspci version. This option should be used stand-alone.

PCILIB AND ITS OPTIONS

The  PCI  utilities  use PCILIB (a portable library providing platform-

independent functions for PCI configuration space access)  to  talk  to

the PCI cards. It supports the following access methods:

linux_sysfs

The  /sys filesystem on Linux 2.6 and newer. The standard header

of the config space is available to all users, the rest only  to

root. Supports extended configuration space and PCI domains.

linux_proc

The  /proc/bus/pci  interface  supported by Linux 2.1 and newer.

The standard header of the config  space  is  available  to  all

users, the rest only to root.

intel_conf1

Direct  hardware  access  via  Intel  configuration mechanism 1.

Available on i386 and compatibles  on  Linux,  Solaris/x86,  GNU

Hurd and Windows. Requires root privileges.

intel_conf2

Direct  hardware  access  via  Intel  configuration mechanism 2.

Available on i386 and compatibles on Linux, Solaris/x86 and  GNU

Hurd.  Requires root privileges. Warning: This method is able to

address only first 16 devices on any bus and it seems to be very

unreliable in many cases.

fbsd_device

The /dev/pci device on FreeBSD. Requires root privileges.

aix_device

Access method used on AIX. Requires root privileges.

nbsd_libpci

The  /dev/pci0  device on NetBSD accessed using the local libpci

library.

By default, PCILIB uses the first available access method and  displays

no  debugging  messages, but you can use the following switches to con-

trol its behavior:

-P <dir>

Force use of the linux_proc access method, using  <dir>  instead

of /proc/bus/pci.

-H1    Use  direct hardware access via Intel configuration mechanism 1.

-H2    Use direct hardware access via Intel configuration mechanism  2.

-F <file>

Extract  all  information  from  given file containing output of

lspci -x. This is very useful for analysis of user-supplied  bug

reports,  because  you can display the hardware configuration in

any way you want without disturbing the user with  requests  for

more dumps.

-G     Increase debug level of the library.

FILES

/usr/local/share/pci.ids

A list of all known PCI ID's (vendors, devices, classes and sub-

classes). Maintained at http://pciids.sourceforge.net/, use  the

update-pciids utility to download the most recent version.

/proc/bus/pci

An  interface  to  PCI  bus  configuration space provided by the

post-2.1.82 Linux kernels. Contains per-bus subdirectories  with

per-card config space files and a devices file containing a list

of all PCI devices.

SEE ALSO

setpci(8), update-pciids(8)

AUTHOR

The PCI Utilities are maintained by Martin Mares <mj@ucw.cz>.

pciutils-2.2.1                 26 November 2005                       lspci(8)